Windows 95 Troubleshooting Guide





�PRIVATE��Question:  Can I still use my 16-bit communications software under Windows 95? 





Answer:  Yes. No additional work is required for your 16-bit communication programs to 


                see the modem under Windows 95. 16-bit applications access the COM Port 


                directly, which Windows 95 natively supports. 	





                �PRIVATE��Note:  If Windows 95 was installed over Windows 3.1, and the COMM.DRV= line in 


                           the Windows SYSTEM.INI file was set to RHSICOM.DRV, you must edit 


                           the SYSTEM.INI file and replace COMM.DRV=COMM.DRV with 


                           COMM.DRV=RHSICOM.DRV. Windows 95 replaces your current 


                           Windows 3.1 COM driver with COMM.DRV during the initial installation.


�





�PRIVATE��Question:  How are 32-bit communication programs handled under Windows 95? 





Answer:  32-bit, or Windows 95 native communication programs, do not directly access 


                COM Ports. They access what is called the TAPI interface, which is a combination 


                of a COM Port, a modem, and a set of dialing rules. To properly set this up, you 


                must run the Modems Control Panel, then ADD the appropriate COM Port and 


                modem combination. This was most likely configured when the modem was 


                initially installed. 





                Once this is complete, HyperTerminal, MSN, Dial-Up networking, and any other 


                Windows 95 native communications applications will be able to address the 


                entire TAPI system. The TAPI system then communicates with the modem, via 


                AT commands.


�





�PRIVATE��Problem:  I configured my modem to COM4/IRQ3, but it doesn't seem to work. 





Solution:  Go into the Windows 95 Control Panel --> System --> Performance-->Graphics. Set 


                 Hardware Acceleration to None. We have found there to be an addressing conflict 


                 with certain graphic accelerator cards. If you configure your Windows 95 graphic 


                 driver to basic VGA, and find the modem now works at that setting, then the 


                 problem is your graphics card. 





                 You may want to try using one of the more commonly used Com Port and IRQ 


                 settings such as:





�PRIVATE��Com 1 �
Irq 4 �
�
Com 2 �
Irq 3 �
�
Com 3 �
Irq 5 (if not used by your sound card)�
�
�


�
�PRIVATE��Problem:  When auto-detect tries to add a new modem at COM 2, Windows 95 locks up. 





Solution:  Open up Control Panel/System Settings and deselect COM2. This can be 


                 accomplished by selecting COM2 under System Settings, then choosing 


                 Properties. There should be a red 'x' in a box down towards the bottom of the 


                 Properties screen. Click once on the red 'x' and it should clear. This disables the 


                 COM Port in Windows 95. Click OK, then restart the machine. When Windows 95 


                 restarts, it should now find the COM Port. This technique can apply to all available 


                 COM Ports.


�





�PRIVATE��Problem:  I'm using Windows 95 built in PCMCIA support. HyperTerminal works, but none of 


                 my 16-bit communication programs work. 





Solution:  Using a text editor, edit the SYSTEM.INI file in your Windows directory. Under the 


                 [386Enh] section of your SYSTEM.INI file, change the COMM.DRV line back to its 


                 original COMM.DRV. This line should read comm.drv=comm.drv. Ensure that 


                 there is a line that states device=*vcd. If your SYSTEM.INI has a line that reads 


                 device=*vrdd, place a semicolon in front of it. Your 16-bit applications should 


                 now work.


�





�PRIVATE��Problem:  My modem will not auto-detect under Windows 95. 	





Solution: 	


Select Start/Settings/Control Panel. 


Select Add New Hardware. 


Select Modems. 


Pick the model modem you have (or the closest to it ).





                If your modem is not listed in the Windows 95 modem list, choose the one closest 


                to it. Always check our Sportster file section on our BBS (847-982-5092) of FTP


                Site (ftp.usr.com) for an available INF file. We try to make them available as soon 


                as possible at the release of a new product.�                


                Here are a few examples: 





�PRIVATE��Your Model                               Select 


Sportster 14.4kbps                          Sportster 14.4kbps 


Sportster 14.4kbps Si                      Sportster 14.4kbps 


Sportster 14.4kbps Vi                      Sportster 14.4kbps or download MDMUSRVI.INF. 


Sportster 28.8kbps                          Sportster 28.8kbps 


Sportster 28.8kbps DSVD               Sportster 28.8kbps 


Sportster 28.8kbps Vi                      Sportster 28.8kbps or download MDMUSRVI.INF.


�





�
�PRIVATE��Problem:  I'm having trouble configuring my Sportster Vi under Windows 95. 





Solution:  We have an INF file available for this modem which provides Windows 95 with 


                 the proper identification and installation instructions for the Sportster Vi product 


                 line, released to date. The following procedures are required to set this up: 


	


1.	Download MDMUSRVI.INF from our BBS (847-982-5092) or FTP Site at ftp.usr.com. 


2.	Copy MDMUSRVI.INF to the \Windows\Inf directory.  This is a hidden directory so you will not be able to see it using a directory listing. 


This is accomplished by the following steps: 


a.	Click on the MS-DOS Icon. 


b.	Copy MDMUSRVI.INF to the inf directory: 


example:  copy c:\ql2faxw\inbox\mdmusrvi.inf c:\windows\inf


c.	Type exit and press <enter>.


3.	Assuming the modem is correctly installed in the computer, select Start/Settings/Control Panel/Modems. 


a.	INTERNAL MODEM: You will have to select Add New Hardware in the Control Panel to install support for the new COM Port the modem provides. Do this after the modem has been installed.


Click on Add. 


Click on Next.


Windows 95 will now rebuild the database of modem drivers. This takes about 25 seconds on a IBM 486/66. Windows 95 will then query the modems attached externally and/or internally on the system. After the query process is complete, Windows 95 will show that it found a "Sportster Vi ...." modem.


Click on Next. 


Click on Finish.





If you have an internal modem, make sure that it is configured to a COM Port and Irq that is not being used by another device in the computer.


�





�PRIVATE��Question:  How do I check to see if I have the release version of Windows 95 and not a 


                  beta copy? 





Answer:  Click on Start/Settings/Control Pannel/System. Select the General Tab. The 


                version number will be located under the System section of the screen. If it shows 


                a version number of 4.00.950 or 4.00.950a (service pack 1), then you have the 


                release version of Windows 95. You can also check by right clicking on the 


                My Computer icon and selecting Properties.


�





�PRIVATE��Question:  Is Windows 95 accurate at determining device conflicts? 





Answer:  Yes. By selecting Start/Settings/Control Panel/System/Device Manager, you can 


                view and make changes to your device settings. This screen uses exclamation 


                icons, etc., to point out devices that are conflicting in the system.


�


�
�PRIVATE��Question:  What happens when you put an internal modem on IRQ 3 or 5. Will it just detect 


                   the device or install it? 





Answer:  There are two sides to this question. If another device is using the IRQ you want to 


                use, then the modem, or the other device must be configured to another IRQ. With 


                that out of the way, we can assume there are no IRQ conflicts. After installing the 


                modem, and starting up the computer, Windows 95 will detect and install the modem. 


                If the device is configure for Plug and Play mode, Windows 95 will detect and 


                configure the card automatic. If the device is not Plug and Play, user assistance 


                will be required. If the modem was not detected at all, open the Control Panel  


                and select Add New Hardware.


�





�PRIVATE��Question:  How can I stay updated on new product identifications for my modem? 





Answer:  U.S. Robotics will provides modem identification configurations as they become 


                available.  For each new product release or revision, the .INF file for that device will 


                have to be updated. This update can be as simple as adding what is called a 


                UNIMODEM Id, or as complex as writing a new .INF file to support advanced 


                features. As a rule, your current .INF file should work just fine.


�





�PRIVATE��Question:  Can you reserve an IRQ to prevent Windows 95 from assigning it to a Plug 


                  and Play device? 





Answer:  You can reserve an IRQ for hardware in Windows 95, but it must be done manually. 


                If the IRQ is in use, Windows 95 will not let you reserve it. To reserve an IRQ, follow 


                these steps: 


[Numbers and periods reversed]


Select Start/Settings/Control Panel/System. 


On the Device Manager tab, double-click Computer. 


Select the Reserve Resources tab and then the Interrupt Request (IRQ) radial button. 


Click Add. 


In the Value box, scroll to the IRQ you want to reserve. 


Click OK until you return to the Control Panel. 


�





�PRIVATE��Question:  Why won't Hyperterminal send commands to the terminal? 





Answer:  There is a new version of Hyperterminal available on our BBS at (847)982-5092 or 


                FTP Site at ftp.usr.com, which corrects this problem.


�


�
�PRIVATE��Question:  What is Safe Mode and why can't I do anything in it? 





Answer:  Windows 95 can load into Safe Mode in two ways: automatically and by user choice. 


                It will occur automatically if there is something severely wrong with the system. In 


                this case, Windows 95 shuts everything down and loads raw Windows. If you decide 


                to enter Safe Mode manually, press F5 immediately after the “Starting Windows 95” 


                message at bootup. Safe Mode itself does not fix the system problems, but gives you 


                the opportunity to correct problems without affecting anything else.


�





�PRIVATE��Question:  Is it normal for Windows 95 to take a long time to boot, or do I have a problem? 





Answer:  Windows 95 does boot slowly, even slower in Safe Mode. This is due to several 


                tasks running in the background and is perfectly normal.


�





�PRIVATE��Question:  What should I do if my computer automatically boots up in Safe Mode? 





Answer:  Click on Start/Settings/Control Panel/System/Device Manager. From this 


                screen you should be able to see any hardware conflicts. If the problem occurred 


                right after the installation of a modem, check for conflicts and if none are found, 


                call U.S. Robotics Support. If the problem is not directly related to the modem, 


                contact Microsoft or the manufacturer of the device that you feel is conflicting.


�





�PRIVATE��Question:  Does Hyperterminal support Z-Modem?





Answer:  Yes, it supports X/Y/Z-modem, ASCII, and Kermit.


�


�PRIVATE��Question:  Will Windows 95 detect my PCMCIA modem? 





Answer:  Yes. We have found that Windows 95 quickly detects and installs our PCMCIA 


                modems. No cardsoft drivers of any type should be loaded in your CONFIG.SYS 


                or AUTOEXEC.BAT. Windows 95 must use its own drivers. All “card and socket 


                services” exist within the Windows 95 operating system and are native 32-bit 


                drivers. NEVER load Phoenix drivers within Windows 95. It will cause many different 


                problems with your system.


�





�PRIVATE��Question:  What is the (;) with no phone number about? 





Answer:  Windows 95 uses “TAPI”, an intermodem interface which uses what they call 


                staged dialing. It can accept 128 characters, which is more than the modem can 


                handle. Because of this, TAPI uses (;) to break the phone number into manageable 


                parts.  This will only work with modems that support this feature, and Windows 95 


                will ignore the command if the modem does not.


�





�
�PRIVATE��Question:  What happens if I exit Windows 95 without using Shutdown or Restart the 


                   computer? 





Answer:  Although this is not recommened, Windows 95 will usually recover from an 


                unexpected termination of the operating system. However, like Windows 3.1, you 


                may find some fragmentation of your drive, temporary files that were not 


                removed, and garbage characters in the program you were using at the time the 


                system closed.


�





�PRIVATE��Question:  Where can I find the latest, or new .INF file for my modem? 





Answer:  U.S. Robotics creatse .INF files for all of our modems. Check the Sportster File 


                Section on our BBS at (847)982-5092 or FTP Site at ftp.usr.com, for the latest 


                updates to our .INF files for Windows 95.


�





�PRIVATE��Question:  What are the System requirements to run Windows 95? 





Answer:  Microsoft says the minimum requirements are: 486 16mhz or faster with 12 megs 


                of RAM, and a VGA card.


�





�PRIVATE��Question:  What can you not Shut Down using the Windows 95 Close Program Task 


                   Manager? 





Answer:  The System Tray and its functions; clock, items running in the background at all 


                times, some controls, and the PCMCIA card if installed.


�





�PRIVATE��Question:  Is DOS still running with Windows 95? 





Answer:  Windows 95 is it's own operating system with an integrated operating environment. 


                In Windows 95, DOS is more emulated than actual DOS. It uses a new version of 


                DOS known by some as DOS 7.0 or DOS 95. This emulated version of DOS 


                integrates better with Windows 95 than running over DOS 6.22 as Windows 3.1 


                does.


�





�PRIVATE��Question:  Does Windows 95 work well with DOS? 





Answer:  Yes, Windows 95 handles normal application and multi-tasking much better then 


                it’s predecessor. The DOS shell still does not run as fluidly as native DOS.


�





�PRIVATE��Question:  Can you write directly to the hardware like in DOS? 





Answer:  No, not in Windows 95. However, even though you have limited functionality, this 


                is possible in Safe Mode.


�


�
�PRIVATE��Question:  Will my Sporster Si work in Windows 95. 





Answer:  Yes. Start by manually install the modem as a Standard 14,400 modem off the 


                picklist in Windows 95. Next, load the WinRPI driver off the Quicklink II disk. After 


                loading the driver, edit the Windows SYSTEM.INI file and ensure the COMM.DRV 


                line reads comm.drv=wrpi.drv. 


	


                When configuring your communication programs, an init string of 


                AT&F+H11&C1&D2S7=90 should work in most cases. This will load the software 


                based compression and error correction. 


U.S. Robotics	Troubleshooting/Windows 95





Page � PAGE �7�	document #11195  � DATE  \l �6/18/1997�











