	USB Network Interface
Model. 3C19250 

Specifications 

 

USB Interface
Universal Serial Bus Specification 1.0 compliant
USB type B connector for standard shielded USB cable, maximum length 5 m
Ethernet Interface
ISO 8802/3, IEEE 802.3 compliant, RJ-45 connector for UTP or STP cable (Category 3 or 5), maximum length 100 m
Throughput
Up to 6 Mbps (sustained)
MTBF
In excess of 1 million hours (100 years)
Physical Dimensions
25 mm x 55 mm x 68 mm (1 in x 2.2 in x 2.7 in)
Operating Temperature
0 to 50°C
 

 

Humidity
10 to 90% noncondensing
Storage Temperature
-40° to 70°C
Storage Humidity
Up to 95% relative humidity
Operating Voltage
5 V (USB standard)
Operating Current
140 mA
Compliance
Safety Agency Certification
UL 1950, EN 60950, CSA 22.2 No. 950
EMC Emissions
ICES-003 Class B, FCC Part 15 Class B, EN 55022 Class B, VCCI Class B (Japan), As/NZS 3548 Class B, CISPR Class B, CNS 13438 Class B
 

Shielded cables must be used to ensure compliance with the Class B requirements of these standards. The use of unshielded cables complies with Class A requirements.
Compatibility
Ethernet half-duplex mode
Promiscuous mode is not supported.
Package Contents
· USB Network Interface 

· USB cable - 0.6 m (2 ft) 

· Installation diskette 

· Installation guide (in English, French, German, Italian, and Spanish) 

Ordering Information


3Com USB Network Interface 3C19250 



 
 
 




