[image: image1.png]1@&




Camera Application Kit
Bill of Materials
Digi document reference number: 90001092
(Digi International Inc. 2009. All Rights Reserved.

The Digi logo is a registered trademark of Digi International, Inc. 

All other trademarks mentioned in this document are the property of their respective owners. 

Information in this document is subject to change without notice and does not represent a commitment on the part of Digi International. 

Digi provides this document “as is,” without warranty of any kind, expressed or implied, including, but not limited to, the implied warranties of fitness or merchantability for a particular purpose. Digi may make improvements and/or changes in this manual or in the product(s) and/or the program(s) described in this manual at any time. 

This product could include technical inaccuracies or typographical errors. Changes are periodically made to the information herein; these changes may be incorporated in new editions of the publication.
Digi International Inc.
11001 Bren Road East

Minnetonka, MN 55343 (USA)

* +1 877 912-3444 or +1 952 912-3444 

http://www.digi.com

	Description
	Manufacturer reference
	Quantity
	Design references

	Slotted cheese head screw, DIN 84
	Schrauben-Jaeger: AG M2X6, A2
	2
	ME18,ME19

	Flat washer, DIN 125, M2, Nylon

	ETTINGER: 03.03.013
	2
	ME16,ME17

	S-Mount lens holder M12x0,5
	Vision Dimension 301410 (EOL)
	1
	H1

	
2.5mm Mini S-Mount Lens


	Vision Dimension 301389 (EOL)
	1
	H2

	Header, 2x10pin, SMT, RM2.54,
	Imperial Connector Systems Ltd G4R-201-C 
	1
	P1

	Resistor: 0.05R, 2A, 220R/100MHz, EMI Fi
	MURATA ERIE NO. AMERICA INC: BLM21PG221SN1
MURATA ERIE NO. AMERICA INC: BLM21PG221SN1B
MURATA ERIE NO. AMERICA INC: BLM21PG221SN1J
MURATA ERIE NO. AMERICA INC: BLM21PG221SN1D
	1
	L1

	IC: TPS79928, LDO, +2.8V, 200mA
	TEXAS INSTRUMENTS: TPS79928DDCR (T&R 3000) 
TEXAS INSTRUMENTS: TPS79928DDCT (T&R 250) 
	1
	U3

	Capacitor: 1uF, 0603, 6.3V, X5R, 10%, 
	MURATA ERIE NO. AMERICA INC. GRM188R60J105 KA01D (6.3V) 
MURATA ERIE NO. AMERICA INC: GRM188R61A105KA01L (10V) 
MURATA ERIE NO. AMERICA INC: GRM188R61C105KA12D (16V) 
	1
	C176

	Capacitor: 2.2uF, 0805, 6.3V, X7R, 10%, -
	MURATA ERIE NO. AMERICA INC: GRM21BR70J225KA01L
	1
	C175

	Capacitor: 100nF, 0402, 10V, X5R, 10%, 
	AVX Corporation: 0402ZD104KAT2A
YAGEO: CC0402KX5R6BB104                                                        KEMET ELECTRONICS: C0402C104K8PAC                            SAMSUNG: CL05A104KP5NNNC
	23
	C1,C13,C14,C15,C16,C17,C18,C19,C2,C20,C21,C22,C23,C25,C26,C27,C28,C3,C4,C5,C6,C7,C8

	Capacitor: 10nF, 0402, 25V, X7R, 10%,
	AVX Corporation: 04023C103KAT2A
KEMET ELECTRONICS: C0402C103K3RAC                           SAMSUNG: CL05B103KA5NNNC                                                   YAGEO: CC0402KRX7R8BB103
	1
	C174

	Capacitor: 10uF, 0603, 6.3V, X5R, 20%, 
	TDK SYSTEMS: C1608X5R0J106MT
	1
	C24

	Capacitor: 2.2pF, 0402, 50V, COG, 
	AVX Corporation: 04023U2R2BAT2A
MURATA ERIR NO. AMERICA INC: GRM1555C1H2R2BZ01D (T&R 10000)
VENKEL: C0402HQN250-2R2BNE
	4
	C10,C11,C12,C9

	IC: Schmitt-Trigger Buffer, SC70-5
	TEXAS INSTRUMENTS: SN74LVC1G17DCK
TEXAS INSTRUMENTS: SN74LVC1G17DCKR 
NXP/PHILIPS COMPONENTS: 74LVC1G17GW 
	13
	U10,U11,U12,U13,U14,U15,U16,U2,U5,U6,U7,U8,U9

	Flat Ribbon Cable, 20pin, 2xSocket
	ES&S: TK03142
	1
	W1

	Resistor: 0R0, 0603, 1A, -55-+125
	VISHAY: D11/CRCW0603 0R ET1 E3
PANASONIC: ERJ3GEOR003V                                                                        YAGEO: RC0603FR-070RL 
YAGEO: RC0603JR-070RL (5%)
	1
	R15

	Resistor: 10K, 0402, 1%, 63mW, -55-+125
	YAGEO: RC0402FR07-10KL
PANASONIC: ERJ2RKF1002X
VISHAY: D10/CRCW0402 100 10K 1% ET7 E3
	4
	R3,R4,R5,R6

	Resistor: 2K2, 0402, 1%, 63mW, -55-+125
	YAGEO: RC0402FR-072K2L
PANASONIC: ERJ2RKF2201X
VISHA:Y D10/CRCW0402 100 2K2 1% ET7E3
	2
	R1, R2

	MT9V111, Digital Image Sensor
	CAM_APKIT_JS_0:Aptina MT9V111I99STC (EOL)
3001050x-01_A: Aptina MT9V111IA7ATC
	1
	U1

	MAX3373, I2C LV Level Translator
	MAXIM MAX3373EEKA+ 
	1
	U4


