Solution Guide

Digi 6330-MX to Cisco ASA IPSec VPN Tunnel using OpenSSL certificates.

Digi Support September 2020

# Contents

1	Introduction	3

1.1	Outline	3

1.2	Assumptions	3

1.3	Corrections	3

2	Version	3

3	Certificates creation	4

If you already have certificates available, you can skip to section 3.2	4

3.1	Generate Test certificates using OpenSSL and XCA	4

3.1.1	Create a Root CA Certificate	4

3.1.2	Create a CA-Signed Host Certificate (Cisco ASA, Responder)	7

3.1.3	Create a CA-Signed Client Certificate (Digi 6330-MX, initiator)	9

3.1.4	Export the certificates and keys in .PEM format	11

4	Digi 6330-MX configuration	14

4.1	Upload SSL certificates to the Digi 6330-MX (initiator)	14

4.1.1	Upload the certificates via the Web GUI	14

4.2	Configure the VPN Tunnel settings on the Digi (Initiator).	14

5	Cisco configuration	19

5.1	Import the certificates and private key	19

5.1.1	Create a trustpoint for the CA root certificate via ASDM and import the CA root certificate in the created Trustpoint with copy and paste	19

5.1.2	Create a Trustpoint for the identity certificate and import the public certificate and the private key in the created Trustpoint with a PKCS#12	20

5.2	Configure the tunnel	20

6	Testing	24

6.1	Confirm Traffic Traverses the IPSec Tunnels	27

7	Configuration files	28

# Introduction

## Outline

This document describes how to create, upload SSL certificates and configure Digi 6330-MX and Cisco routers to build an IPsec VPN tunnel.

## Assumptions

This guide has been written for use by technically competent personnel with a good understanding of the communications technologies used in the product and of the requirements for their specific application. It also assumes a basic ability to access and navigate a Digi router.

This application note applies only to:

Model: DIGI 6330-MX running 20.5.38.58 and later

Model: Cisco ASA running 9.12 Image.

## Corrections

Requests for corrections or amendments to this application note are welcome and should be addressed to: tech.support@digi.com

Requests for new application notes can be sent to the same address.

# Version

| Version Number | Status |
| --- | --- |
| 1.0 |  |

# certificates creation

## If you already have certificates available, you can skip to section 3.2

## 3.1 Generate Test certificates using OpenSSL and XCA

Download and install the latest release of XCA which can be found at: http://sourceforge.net/projects/xca/

### 3.1.1   Create a Root CA Certificate

Open the XCA application

- Click the File menu and select New Database, chose a name and click Save.

- Set up a password and click OK

- Click the Certificates tab

- Click the New Certificate button

- Under “Template for the new certificate”, select default CA and click Apply all

- Go to the Subject tab, fill in all the information then click the Generate a new key button and click OK

| Parameter | Setting |
| --- | --- |
| Internal name | This is for display purposes in the tool, only |
| Country Name | The two-letter ISO 3166 abbreviation for your country. |
| State or Province Name | The state or province where your organization is legally located. Do not abbreviate. In this example: Some-State |
| Locality Name | The city where your organization is legally located. Do not abbreviate. In this example: Paris |
| Organization Name | The exact legal name of your organization. Do not abbreviate your organization name. In this example: Digi |
| Organizational Unit Name | Section of the organization. Examples of sections are Marketing, Research and Development, Human Resources or Sales. |
| Common Name | In this example DigiCA will be used. |
| Email Address | Enter your organization general email address. In this example certteam@digi.com |

- The certificate should now appear in the window with the CA : YES confirmation. If it does not say CA: YES, verify that you selected CA in the template and clicked Apply All.

### 3.1.2 Create a CA-Signed Host Certificate (Cisco ASA, Responder)

- Click the Certificates tab

- Click the New  Certificate button

- Under Signing, make sure to select “Use this Certificate for signing” and chose the previously created CA.

- Under “Template for the new certificate”, select default HTTPS_server and click Apply all

- Go to the Subject tab, fill in all the information then click the Generate a new key button and click OK

| Parameter | Setting |
| --- | --- |
| Internal name | This is for display purposes in the tool, only |
| Country Name | The two-letter ISO 3166 abbreviation for your country. |
| State or Province Name | The state or province where your organization is legally located. Do not abbreviate. In this example: Some-State |
| Locality Name | The city where your organization is legally located. Do not abbreviate. In this example: Munich |
| Organization Name | The exact legal name of your organization. Do not abbreviate your organization name. In this example: DigiDE |
| Organizational Unit Name | Section of the organization. Examples of sections are Marketing, Research and Development, Human Resources or Sales. |
| Common Name | In this example 6330mx will be used. This will be used as the router Identity for the IPSec tunnel settings |
| Email Address | Enter your organization general email address. In this example digide@digi.com |

- The certificate should now appear in the window under the CA certificate.

### 3.1.3 Create a CA-Signed Client Certificate (Digi 6330-MX, initiator)

- Click the Certificates tab

- Click the New  Certificate button

- Under Signing, make sure to select “Use this Certificate for signing” and chose the previously created CA.

- Under “Template for the new certificate”, select default HTTPS_client and click Apply all

- Go to the Subject tab, fill in all the information then click the Generate a new key button and click OK

| Parameter | Setting |
| --- | --- |
| Internal name | This is for display purposes in the tool, only |
| Country Name | The two-letter ISO 3166 abbreviation for your country. |
| State or Province Name | The state or province where your organization is legally located. Do not abbreviate. In this example: Some-State |
| Locality Name | The city where your organization is legally located. Do not abbreviate. In this example: Munich |
| Organization Name | The exact legal name of your organization. Do not abbreviate your organization name. In this example: DigiDE |
| Organizational Unit Name | Section of the organization. Examples of sections are Marketing, Research and Development, Human Resources or Sales. |
| Common Name | In this example wrdigide will be used. This will be used as the router Identity for the IPSec tunnel settings |
| Email Address | Enter your organization general email address. In this example digide@digi.com |

- The certificate should now appear in the window under the CA certificate.

### 3.1.4 Export the certificates and keys in .PEM format

- Select the Certificates Tab.

- Highlight the CA certificate and click the Export button

- In the Certificate export window, select PEM as the export format and click OK

- Repeat the previous step for the Digi router certificate.

- Select the Private Keys tab.

- Highlight the Digi  6330-MX certificate and click the Export button

In the Key export window, select PEM as the export format and click OK

- Repeat the previous step for the Cisco ASA certificate.

Please note: Cisco ASA firewall requires the certificate to be concatenated with encrypted key in format PKCS#12.  Make sure to check the export format as encrypted PKCS#12.

The following files should now be available:

- CA.crt : CA root certificate

- asa.p12 : Cisco ASA (responder) certificate with encrypted private key

- 6330mx.crt : Digi 6330-MX (initiator) certificate

- 6330mx.pem : Digi 6330-MX (initiator) private key

# Digi 6330-MX configuration

## Upload SSL certificate to the Digi 6330-MX (initiator)

### Upload the certificates via the Web GUI

Open a web browser to the IP address of the Digi router 6330-MX (initiator)

System > Configuration > VPN > IPSec > Tunnels > Tunnel name > Authentication

Click on the drop down menu Authentication type and select X.509 certificate option.

Then open Certificate and Private key files (6330mx.crt, 6330mx.pem) with any text editor and copy/paste all content of the files to the corresponding configuration fields.

Click on the drop down menu Peer verification and select Certificate Authority option.

Then copy and paste content of the file CA.crt to the Certificate Authority chain field.

## Configure the VPN Tunnel settings on the Digi 6330-MX (Initiator).

Follow the Web UI interface path:

System > Configuration > VPN > IPSec > Tunnels

Add a new IPSec tunnel with the appropriate configuration as indicated on the screenshot below:

To avoid a problem with network address translation, when cellular provider does not allocate a public IP addresses to the clients, an UDP encapsulation was enforced in the settings.

| Parameter | Setting | Description |
| --- | --- | --- |
| Description | to_asa | Description of the IPsec tunnel |
| Remote endpoint IP Address / Hostname | asa5506.digi.com | IP Address or hostname of the remote endpoint router (responder) |
| Local Network | 192.168.20.0/24 | Local Lan IP address |
| Remote Network | 192.168.25.0/24 | Remote Lan IP address |
| Local endpoint Type | Default route | The method of determining the local network interface that is used to communicate with the peer |
| ID Type | Raw | The type of identifier to be used |
| Local endpoint ID value | 6330mx | ID that is matching the CN of the certificate in the Digi router (initiator) |
| Local network Type | Custom network | The method for determining the local network |
| Remote endpoint ID value | asa5506.digi.com | Remote ID that is matching the CN in the Cisco ASA firewall certificate (responder) |
| Phase 1 lifetime | 3 hours | The period of time after a successful negotiation that the IKE security association expires and must be reauthenticated |
| Phase 2 lifetime | 1 hour | The period of time after a successful negotiation that the IPSec security association expires and must be rekeyed |
| Lifetime margin | 9 minutes | The amount of time before the end of the Phase 1 and Phase 2 lifetimes that renegotiation may be initiated |
| IKE version | 1 | IKE protocol version used to setup the tunnel |
| Enable padding | disabled | Enable padding of IKE packets to 4 bytes |
| Initiate connection | enable | Initiate the key exchange, rather than waiting for an incoming request |
| Mode | Main mode | The IKE Phase 1 mode determines how to establish a secure channel between the peers for the further negotiation |
| NAT Destination network | 192.168.25.0/24 | The destination network that requires source NAT |
| Dead peer detection | enable | Dead peer detection uses periodic IKE transmissions to the remote endpoint to detect whether tunnel communications have failed |

| Parameter | Setting | Description |
| --- | --- | --- |
| Phase 1 Cipher | AES (256 bit) | Encryption settings used on the Phase 1 |
| Phase 2 Cipher | AES (256 bit) | Encryption settings used on the Phase 2 |
| Phase 1 Hash | SHA1 | The Hash to use for checking communication integrity on Phase 1 |
| Phase 1 Hash | SHA1 | The Hash to use for checking communication integrity on Phase 2 |
| MODP Group for Phase 1 | 2 (1024) | DH Phase 1 |
| MODP Group for Phase 2 | 2 (1024) | DH Phase 2 |

Click Apply to save the settings.

# Cisco ASA configuration

## The values for Date, Time, and Time Zone must be accurate in order for the proper certificate validation to occur.

## Import the certificates and private key

### Create a Trustpoint for the CA root certificate via ASDM and import the CA root certificate in the created Trustpoint with copy and paste

### Create a Trustpoint for the identity certificate and import the public certificate and the private key in the created Trustpoint with a PKCS#12 file.

## Configure the tunnel

Enable outside interface for IPSec access:

Create an IKEv1 policy

Create IKEv1 IPSec proposal:

Configure IPSec Site-to-Site Tunnel group:

Configure Crypto Map:

Create certificate to Connection profile maps rule:

The Cisco ASA is now configured and the tunnel should come up.

# Testing

This section will show that the IPSec tunnel has been established.

Cisco ASA

asa5506# sh crypto isa sa detail

IKEv1 SAs:

Active SA: 1

Rekey SA: 0 (A tunnel will report 1 Active and 1 Rekey SA during rekey)

Total IKE SA: 1

1   IKE Peer: 95.115.25.102

Type    : L2L             Role    : responder

Rekey   : no              State   : MM_ACTIVE

Encrypt : aes-256         Hash    : SHA

Auth    : rsa             Lifetime: 10800

Lifetime Remaining: 9552

There are no IKEv2 SAs

asa5506# sh crypto ipsec sa peer 95.115.25.102

peer address: 95.115.25.102

Crypto map tag: finsing, seq num: 5, local addr: 37.81.85.5

access-list outside_cryptomap extended permit ip 192.168.25.0 255.255.255.0 192.168.20.0 255.255.255.0

local ident (addr/mask/prot/port): (192.168.25.0/255.255.255.0/0/0)

remote ident (addr/mask/prot/port): (192.168.20.0/255.255.255.0/0/0)

current_peer: 95.115.25.102

#pkts encaps: 0, #pkts encrypt: 0, #pkts digest: 0

#pkts decaps: 0, #pkts decrypt: 0, #pkts verify: 0

#pkts compressed: 0, #pkts decompressed: 0

#pkts not compressed: 0, #pkts comp failed: 0, #pkts decomp failed: 0

#pre-frag successes: 0, #pre-frag failures: 0, #fragments created: 0

#PMTUs sent: 0, #PMTUs rcvd: 0, #decapsulated frgs needing reassembly: 0

#TFC rcvd: 0, #TFC sent: 0

#Valid ICMP Errors rcvd: 0, #Invalid ICMP Errors rcvd: 0

#send errors: 0, #recv errors: 0

local crypto endpt.: 37.81.85.5/4500, remote crypto endpt.: 95.115.25.102/4500

path mtu 1500, ipsec overhead 82(52), media mtu 1500

PMTU time remaining (sec): 0, DF policy: copy-df

ICMP error validation: disabled, TFC packets: disabled

current outbound spi: CE754E6D

current inbound spi : 0BF0F91E

inbound esp sas:

spi: 0x0BF0F91E (200341790)

SA State: active

transform: esp-aes-256 esp-sha-hmac no compression

in use settings ={L2L, Tunnel,  NAT-T-Encaps, PFS Group 2, IKEv1, }

slot: 0, conn_id: 2331, crypto-map: finsing

sa timing: remaining key lifetime (kB/sec): (3915000/2320)

IV size: 16 bytes

replay detection support: Y

Anti replay bitmap:

0x00000000 0x00000001

outbound esp sas:

spi: 0xCE754E6D (3463794285)

SA State: active

transform: esp-aes-256 esp-sha-hmac no compression

in use settings ={L2L, Tunnel,  NAT-T-Encaps, PFS Group 2, IKEv1, }

slot: 0, conn_id: 2331, crypto-map: finsing

sa timing: remaining key lifetime (kB/sec): (3915000/2320)

IV size: 16 bytes

replay detection support: Y

Anti replay bitmap:

0x00000000 0x00000001

Digi 6330-MX

# ipsec status

Shunted Connections:

Bypass LAN 127.0.0.0/8:  127.0.0.0/8 === 127.0.0.0/8 PASS

Bypass LAN 192.168.1.0/24:  192.168.1.0/24 === 192.168.1.0/24 PASS

Bypass LAN 192.168.1.1/32:  192.168.1.1/32 === 192.168.1.1/32 PASS

Bypass LAN fe80::ce32:e5ff:fe59:f2a9/128:  fe80::ce32:e5ff:fe59:f2a9/128 === fe80::ce32:e5ff:fe59:f2a9/128 PASS

Bypass LAN fe80::/64:  fe80::/64 === fe80::/64 PASS

Bypass LAN 169.254.0.0/16:  169.254.0.0/16 === 169.254.0.0/16 PASS

Bypass LAN 192.168.20.0/24:  192.168.20.0/24 === 192.168.20.0/24 PASS

Bypass LAN 192.168.210.0/24:  192.168.210.0/24 === 192.168.210.0/24 PASS

Bypass LAN fd00:2704::/64:  fd00:2704::/64 === fd00:2704::/64 PASS

Security Associations (1 up, 0 connecting):

to_asa_1of1[155]: ESTABLISHED 17 minutes ago, 192.168.1.119[6330mx]...37.81.85.5[asa5506.digi.com]

to_asa_1of1{128}:  INSTALLED, TUNNEL, reqid 1, ESP in UDP SPIs: ce754e6d_i 0bf0f91e_o

to_asa_1of1{128}:   192.168.20.0/24 === 192.168.25.0/24

# ip -s xfrm state

src 192.168.1.119 dst 37.81.85.5

proto esp spi 0x0bf0f91e(200341790) reqid 1(0x00000001) mode tunnel

replay-window 0 seq 0x00000000 flag af-unspec (0x00100000)

auth-trunc hmac(sha1) 0xa69f3c0555d8f899e7124297ca6e3f2746509fc5 (160 bits) 96

enc cbc(aes) 0xdff342216a6143a80f63bfa24f61b3eab1b223619dbeb8b15507051506055335 (256 bits)

encap type espinudp sport 4500 dport 4500 addr 0.0.0.0

anti-replay context: seq 0x0, oseq 0x0, bitmap 0x00000000

lifetime config:

limit: soft (INF)(bytes), hard (INF)(bytes)

limit: soft (INF)(packets), hard (INF)(packets)

expire add: soft 2954(sec), hard 3600(sec)

expire use: soft 0(sec), hard 0(sec)

lifetime current:

0(bytes), 0(packets)

add 2020-09-10 17:20:03 use -

stats:

replay-window 0 replay 0 failed 0

src 37.81.85.5 dst 192.168.1.119

proto esp spi 0xce754e6d(3463794285) reqid 1(0x00000001) mode tunnel

replay-window 32 seq 0x00000000 flag af-unspec (0x00100000)

auth-trunc hmac(sha1) 0x766a9dccbfa77a94d1c150a7c84620c835430871 (160 bits) 96

enc cbc(aes) 0x1b2cfb289992da38288f3e86af505256bc66824f15e333b425c1ba8641c5c2b5 (256 bits)

encap type espinudp sport 4500 dport 4500 addr 0.0.0.0

anti-replay context: seq 0x0, oseq 0x0, bitmap 0x00000000

lifetime config:

limit: soft (INF)(bytes), hard (INF)(bytes)

limit: soft (INF)(packets), hard (INF)(packets)

expire add: soft 2935(sec), hard 3600(sec)

expire use: soft 0(sec), hard 0(sec)

lifetime current:

0(bytes), 0(packets)

add 2020-09-10 17:20:03 use -

stats:

replay-window 0 replay 0 failed 0

## Confirm Traffic Traverses the IPSec Tunnels

This section will show traffic passing across the tunnel. To test this easily, an ICMP Echo Request/Reply (or PING) will pass from the Digi router 6330-MX  lan (initiator) to Cisco ASA firewall Ethernet interface side (responder)

#  ping 192.168.25.1

PING 192.168.25.1 (192.168.25.1) 56(84) bytes of data.

64 bytes from 192.168.25.1: icmp_seq=1 ttl=255 time=118 ms

64 bytes from 192.168.25.1: icmp_seq=2 ttl=255 time=76.6 ms

64 bytes from 192.168.25.1: icmp_seq=3 ttl=255 time=125 ms

64 bytes from 192.168.25.1: icmp_seq=4 ttl=255 time=72.9 ms

64 bytes from 192.168.25.1: icmp_seq=5 ttl=255 time=111 ms

64 bytes from 192.168.25.1: icmp_seq=6 ttl=255 time=79.0 ms

^C

--- 192.168.25.1 ping statistics ---

6 packets transmitted, 6 received, 0% packet loss, time 0ms

rtt min/avg/max/mdev = 72.912/97.127/125.054/21.414 ms

asa5506# ping inside 192.168.20.1 repeat 5

Type escape sequence to abort.

Sending 5, 100-byte ICMP Echos to 192.168.20.1, timeout is 2 seconds:

!!!!!

Success rate is 100 percent (5/5), round-trip min/avg/max = 60/82/110 ms

# Configuration files

Digi 6330-MX

auth group admin acl shell enable "true"

auth idle_timeout ""

network interface lan ipv4 address "192.168.20.1/24"

add service dns host end

service dns host 0 address "37.81.85.5"

service dns host 0 name "asa5506.digi.com"

add vpn ipsec tunnel to_asa

vpn ipsec tunnel to_asa auth cert "-----BEGIN CERTIFICATE-----

-----END CERTIFICATE-----"

vpn ipsec tunnel to_asa auth peer_ca "-----BEGIN CERTIFICATE-----

-----END CERTIFICATE-----"

vpn ipsec tunnel to_asa auth peer_verify "ca"

vpn ipsec tunnel to_asa auth private_key "-----BEGIN RSA PRIVATE KEY-----

-----END RSA PRIVATE KEY-----"

vpn ipsec tunnel to_asa auth type "x509"

vpn ipsec tunnel to_asa force_udp_encap "true"

add vpn ipsec tunnel to_asa ike phase1_proposal end

vpn ipsec tunnel to_asa ike phase1_proposal 0 cipher "aes256"

vpn ipsec tunnel to_asa ike phase1_proposal 0 dh_group "modp1024"

add vpn ipsec tunnel to_asa ike phase2_proposal end

vpn ipsec tunnel to_asa ike phase2_proposal 0 cipher "aes256"

vpn ipsec tunnel to_asa ike phase2_proposal 0 dh_group "modp1024"

vpn ipsec tunnel to_asa ipsec_failover "to_asa"

vpn ipsec tunnel to_asa local id raw_id "6330mx"

vpn ipsec tunnel to_asa local id type "raw"

add vpn ipsec tunnel to_asa nat end

vpn ipsec tunnel to_asa nat 0 dst "192.168.25.0/24"

add vpn ipsec tunnel to_asa policy end

vpn ipsec tunnel to_asa policy 0 local custom "192.168.20.0/24"

vpn ipsec tunnel to_asa policy 0 local type "custom"

vpn ipsec tunnel to_asa policy 0 remote network "192.168.25.0/24"

vpn ipsec tunnel to_asa remote hostname "asa5506.digi.com"

vpn ipsec tunnel to_asa remote id raw_id "asa5506.digi.com"

vpn ipsec tunnel to_asa remote id type "raw"

Cisco ASA

crypto ipsec ikev1 transform-set ESP-AES-256-SHA esp-aes-256 esp-sha-hmac

crypto ipsec security-association pmtu-aging infinite

crypto dynamic-map finsing 5 match address outside_cryptomap

crypto dynamic-map finsing 5 set pfs

crypto dynamic-map finsing 5 set ikev1 transform-set ESP-AES-256-SHA

crypto dynamic-map finsing 5 set reverse-route

crypto map outside_map 5 ipsec-isakmp dynamic finsing

crypto map outside_map interface outside

crypto ca trustpoint CA_DE

enrollment terminal

crl configure

crypto ca trustpoint DE

keypair DE

no validation-usage

crl configure

crypto ca trustpool policy

crypto ca certificate map finsing 10

subject-name attr cn eq 6330mx

crypto ca certificate chain CA_DE

certificate ca 61db0ca42be7461f

quit

crypto ca certificate chain DE

certificate 0c22bf3f170cab4c

quit

certificate ca 61db0ca42be7461f

quit

crypto ikev1 enable outside

crypto ikev1 policy 1

authentication rsa-sig

encryption aes

hash sha

group 5

lifetime 86400

crypto ikev1 policy 2

authentication rsa-sig

encryption aes-256

hash sha

group 2

lifetime 86400